HACOUT  ·  IDEATION ROUND  ·  THEME: RENEWABLE ENERGY INTELLIGENCE

GridPulse

AI-Powered Renewable Generation Forecasting Platform

Team [ add team name ]   Users grid operators · utilities · plant owners · energy traders   Horizon 24–72 h, hourly   Status working prototype

Every utility already receives tomorrow's weather forecast. Nothing turns it into tomorrow's power, hour by hour, per site — then flags the hours that breach that site's limits and states the action. GridPulse is that missing conversion layer.

# 1  Problem Statement

Solar and wind output swings constantly with weather, time of day and season. Grid operators must commit capacity, schedule backup and respect grid connection limits hours ahead — every one of those decisions made on a forecast. When the forecast is weak, the cost lands in three places:

- Wasted clean energy. Surplus that could have been stored or sold is curtailed, because nobody saw it early enough to place it.

- Unnecessary fossil backup. Reserve plant is held spinning against uncertainty rather than against a known shortfall.

- Unsized financial exposure. A plant that sold power day-ahead cannot tell, in advance, which hours it will fail to deliver.

The gap is not data — it is the conversion. Weather forecasts are public and free. What no one hands an operator is: this site will make 100.6 MW at 07:00, which is 106% of your substation limit, so curtail 5.6 MW.

# 2  Proposed Solution

GridPulse is an intelligence platform that ingests weather forecasts, historical generation records and site-level parameters, and returns a 24–72 hour generation forecast, flagged risk windows, and an explained grid action for each one.

- Forecast — hourly predicted output per site with an 80% confidence band, derived from the same numerical weather prediction a utility already receives.

- Flag — over-generation, under-generation and ramp windows, measured against that site's real operating limits rather than a generic percentage of nameplate.

- Recommend — curtailment, storage charge or dispatch, backup activation or spinning reserve, each carrying a reason sentence assembled from the numbers behind it.

Exactly one component is learned. The weather-to-power conversion is machine learning, because that relationship is non-linear and worth fitting. Everything after it is deterministic arithmetic an operator can audit line by line — because nobody curtails 100 MW on the grounds that a model said so.

# 3  Approach

| Weather feed Open-Meteo NWP, public, no API key | → irradiance, wind, cloud, temperature | Generation model XGBoost, one per technology | → MW per hour, 72 h, with p10–p90 band | Risk rules over · under · ramp, vs. site limits | → flagged windows and limit crossed | Grid action curtail · store · backup · reserve |
| --- | --- | --- | --- | --- | --- | --- |

The pipeline. One learned component converts weather into power; every decision after it is a comparison between two numbers the dashboard also puts on screen.

Forecast weather, then convert it — rather than predicting power directly. This is how renewable forecasting genuinely works in industry, and it means a site with no operating history is forecastable from its coordinates alone.

Build the training history from physics — no utility hands out SCADA data during a hackathon, and random numbers teach a model nothing. We replay 15 months of real ERA5 reanalysis weather per site through first-order generation physics — a PV performance-ratio model (PR 0.78, −0.4%/°C above 25 °C) and a four-region turbine power curve (cut-in 3.5, rated 12.5, cut-out 25 m/s) — plus 6% stochastic noise. That yields 65,808 weather-correlated site-hours carrying the diurnal cycle, seasonal swing, cloud events and storm shutdowns a model needs to learn. Swapping in real SCADA is a file change, not a redesign.

Predict capacity factor, not kilowatts — so one model per technology serves a 10 MW rooftop and a 240 MW offshore array alike, error reads directly as a percentage of capacity, and adding a new site requires no retraining.

Run the horizon recursively — lagged generation is real signal and we do not know next Tuesday's output, so each predicted hour feeds the next as its lag. Confidence bands come from held-out residuals bucketed by predicted output, widened with lead time.

Prove it honestly — a chronological holdout — never shuffled, which would let the model see 13:00 and 15:00 while predicting 14:00 — scored against the persistence baseline a control room actually falls back on, plus a rolling day-ahead backtest in which the model is rewound each morning and never shown the answers.

## Validation results

| Metric (held-out data, never seen in training) | Solar | Wind |
| --- | --- | --- |
| Mean absolute error, % of nameplate capacity | 0.97% | 1.43% |
| Root mean squared error, % of capacity | 1.92% | 2.28% |
| R² | 0.9931 | 0.9932 |
| Persistence baseline MAE (what a control room does without a model) | 5.24% | 5.94% |
| Improvement over that baseline | 81.4% | 75.9% |
| Held-out hours scored | 6,579 | 6,579 |

How to read these numbers. R² is 0.993 for both models because the training targets come from a physics model plus noise, so the mapping being learned genuinely exists in the data. Against real SCADA, day-ahead solar forecasts typically land nearer 4–8% MAE — real plants also have outages, soiling and curtailment no weather feed predicts. Read these as evidence the model has correctly learned the weather-to-power relationship. The figure that transfers is the persistence comparison.

# 4  Key Features

## Portfolio view

- World map with a pin per site, colour-coded by current alert status.

- KPIs: capacity online, generating now, forecast for the next 24 h, sites needing action, and how much of the at-risk curtailment on-site storage can actually absorb.

- A cross-portfolio action queue ranked by urgency, so one screen covers the whole estate.

## Site detail

- 72-hour forecast chart with a shaded confidence band, recorded output stitched to the forecast across a NOW marker, and each flagged risk window shaded on the chart itself rather than listed separately.

- Operating limits drawn as reference lines — export limit, over-generation trigger, under-generation floor — so a breach is visible, not inferred.

- A “yesterday's forecast vs actual” panel scored in MAE, RMSE and R², plus the weather driving the prediction: irradiance, cloud cover, hub-height wind and temperature.

## Flagging that reflects real constraints

- Over-generation — above 85% of the site's grid export limit. A plant that can make 180 MW behind a 95 MW substation is over-generating at 81 MW, whatever headroom the panels still have. Connection limits cause most real curtailment.

- Under-generation — below the greater of 15% of capacity and the site's firm delivery schedule. A plant that sold 45 MW day-ahead is short at 30 MW even though 30 MW is healthy output. That is the number a trader is exposed on.

- Ramp events — swings past 25% of capacity in an hour, merged into single events so a front crossing a wind farm is one thing to schedule against, not four separate alerts.

## Recommendations that explain themselves

- Storage handling is quantitative, not a lookup: the engine sizes the surplus against real battery headroom, tracks state of charge forward through the horizon, and where the battery covers only part of the surplus recommends both actions — charge what fits, curtail the rest.

- Every sentence is assembled from live figures, so the wording moves with the forecast. Nothing in the interface is hardcoded demo text.

“Forecast peaks at 100.6 MW across 4h (06:00–09:00 UTC on 13 Sep), 106% of the 95.0 MW substation export limit. The 60 MWh battery is already at 100% and has no headroom left, leaving 9.2 MWh with nowhere to go — issue a curtailment instruction capped at 95.0 MW export, trimming up to 5.6 MW at the peak hour.” VERBATIM FROM GET /api/sites/{id}/recommendations

# 5  Technology and Implementation Plan

| Backend | Python · FastAPI · SQLAlchemy + SQLite · pandas · APScheduler for scheduled forecast refresh |
| --- | --- |
| Machine learning | XGBoost gradient-boosted regressors, one per technology · scikit-learn for metrics · 20 engineered features per hour, including cyclical time encoding, NOAA solar geometry, clear-sky index, wind shear and generation lags |
| Frontend | Next.js 15 · TypeScript · Tailwind CSS · Recharts · react-leaflet · Framer Motion |
| Data sources | Open-Meteo forecast API and ERA5 reanalysis archive API — free, keyless, no signup, no rate-limit management |
| Serving | 9 REST endpoints; each site's forecast computed once and cached, refreshed on a schedule and warmed at startup, so the dashboard answers in 10–40 ms |

No paid APIs and no service requiring a credit card or an approval wait. The whole system runs from a clean clone with documented install commands and an empty .env file.

## Build sequence

| Stage | Deliverable | Status |
| --- | --- | --- |
| Data layer | Open-Meteo client, PV and turbine physics, 15-month historical dataset builder | Complete |
| ML pipeline | Feature engineering shared by training and serving, chronological training, metrics written to file | Complete |
| API + rules | Sites, weather, forecast, backtest, alerts, recommendations, portfolio summary; flagging and recommendation engines | Complete |
| Dashboard | Portfolio map and KPIs, site detail view, backtest panel, model report | Complete |
| Next | Quantile regression for calibrated p10/p50/p90 bands; multi-model NWP ensemble, using cross-model spread as its own uncertainty signal | Planned |
| Then | SCADA ingestion replacing the synthetic history; Postgres + TimescaleDB at fleet scale; price-aware curtailment economics; scheduled retraining with drift monitoring | Planned |
| Later | Write-back into the dispatch system behind an operator sign-off step, so a recommendation becomes an instruction | Planned |

Prototype already running. Six real sites totalling 892 MW across four continents — three solar, three wind — forecasting on live weather, with the flagging and recommendation engines operating end to end. The architecture above is not drawn on paper; each remaining row is a swap behind an interface that already exists.